Birla Institute Of Technology and Sciences, Pilani

CS F241-Microprocessor Programming and Interfacing

P26-Elevator Control

Submitted to-

## Professor G Sai Sesha Chalapathi

## Submitted by-

## Rohit Lodha (2015A7PS040P)

## Vikram Nitin(2015A7PS042P)

## Aayushmaan Jain(2015A7PS043P)

## Saurabh Raje(2015A7PS045P)

ELEVATOR CONTROL

SYSTEM REQUIREMENTS

- The elevator operates along 3 floors.

- When not in use the elevator is always on the ground floor.

- The elevator can be called by pressing any one of two buttons available on each floor.

  - One button is up and the other is down.

- Whether the elevator stops at the floor or not depends on the direction in which the lift moves. For eg: if the lift is moving in upward direction and the person on say the 2nd floor presses the down button; the lift will not stop in the current journey. When the lift reaches the 3rd floor and starts moving down then the lift will stop at the 2nd floor.

- At every floor there is a 7-segement display that indicates the floor in which the lift is right now. The display can be any value from 0 - 3. ‘0’ indicates the ground floor.

- Inside the lift - buttons are available for floor selection.

- The floor towards which the lift is moving is also displayed within the lift.

- Doors to the lift open and close automatically.

- When the lift reaches any floor where it has to stop it opens automatically, and it closes when a button called “Door Close” is pressed. Lift does not move until the door is closed.

- System runs from a standard power inlet.

SYSTEM SPECIFICATIONS

- An Electro-magnetic system is used for open and close of the door. We have just provided the on/off control.

- A heavy duty servo motor is used for lift movement. We have just provided the input to the driver circuit.

- The inputs are direction (up/down) and a PWM input which control the speed at which the lift moves. The duty cycle can vary from 20% to 60%.

- The frequency of the PWM signal is 20 Hz.

- For detecting whether the lift has reached a floor, the system has a set of three sensors –

  - Two ‘coarse’ sensors and a ‘fine’ sensor. All the sensors are contact switches (i.e.) when the lift reaches the point where the sensors are placed, the contact switch gets pushed in. Output of contact switches are low when closed and high otherwise. The sensor arrangement is represented in the fig below

- On the ground floor – only Coarse Sensor1 and Fine Sensor will be available. On the 3rd floor only Coarse Sensor 2 and the Fine Sensor will be available.

- When the lift starts at the ground floor it starts at a low speed gradually accelerating to the maximum speed. It should operate at maximum speed when it reaches ‘Coarse Sensor 1”. As the lift moves up if it has to stop at floor ‘1’, when Coarse Sensor 2 is detected at that floor the lift starts moving at a low speed until it can stop when it reaches Fine sensor. When it starts again it moves at low speeds and reaches the maximum possible speed when it reaches the fine sensor. The same is done in the reverse direction with the appropriate sensors.

- Speed at which the lift moves is proportional to the duty cycle. For acceleration, duty cycle has to be gradually increased from 20 % to 60 %. And for deceleration, the duty cycle reduced from 60 % to 20 %. The increase is in steps of 20 %

- A 7447 chip (BCD to seven segment converter) is used for driving the 7-segment displays.

- 7447 takes a 4-bit BCD value and converts into the corresponding 7-segement equivalent.

ASSUMPTIONS

- The Program starts at 0100H when the processor is reset.

- Coarse and fine sensors that are available on each floor produce a binary output. Each sensor (coarse and fine) is assumed to be a push button, pushed by the elevator as it moves.

- Coarse and fine sensors have to be manually pressed during the simulation. (However, in reality they would be automatically pressed by the contact from the elevator, as it moves.)

- Floor buttons – both in and outside the life – have been assumed to be push buttons, as well. The floor buttons are pressed by the user.

- While a button is not being pressed, it generates a logic 0 since it has been pulled down. This has been done to prevent the input from floating.

- Unless the door is closed manually, the lift will not move.

- Only one button is pressed at a given instant. However, any button on the floor may be pressed while the elevator is in motion.

- By default the elevator is on the ground floor, and awaits user input.

- Uni-directional motors have been used in this design project. Thus, to simulate both the upward and downward motion, two motors have been used – one for taking the lift up and one for taking it down.

- PWM (Pulse Width Modulation) input is given through a pulse generator.

- An Electro-magnetic system is used for opening and closing of the door. We have assumed that the doors open automatically, and have just provided a push button for closing the elevator door (just an on/off mechanism).  We have also assumed that some mechanism exists that takes the door close button input and closes the door.

- We have assumed that some mechanism involving a heavy duty servo motor already exists, and we have just provided an input to its driver circuit.

- In the physical implementation, there will be separate sets of sensors for each floor. However, we have simplified the design to have just one set of sensors (one fine and two coarse). Since any two sets of sensors will never be triggered together, this is a reasonable simplification. These sensors can be used to accelerate or decelerate the lift, and the number of times the sensors are triggered can be used to infer the floor on which the lift is.

Hardware Description

- Two 8255 chips have been used – one having start address as 00H and the other as 10H.

  - The first 8255 chip has been used to interface the two motors – via a series of resistors. Two LEDS which indicate the status of the coarse sensors have also been connected to this chip. Its Port A and Port C are used for controlling the motors, and Port B has indicator LEDs for the contact sensors.

  - The second 8255 chip has been used to interface the 7-segment displays as well as the various (input) buttons and the coarse and fine sensors. Its Port A is used for controlling the seven segment displays, and Port C is used to access the sensor data. Port B is unused.

- 4KB of RAM has been used in this project – two 6116 chips. The RAM has been divided into two banks (odd and even) of 2KB each. The starting address of the RAM is 01000H.

- 4KB of ROM has been used in this project – two 2716 chips. The ROM has also been divided into two banks (odd and even) of 2KB each. The starting address of the ROM used is 02000H.

- 4 seven segment displays, one on each floor, indicate the floor the elevator is currently at.

  - One seven segment display inside the lift indicates the target floor (the floor to which the elevator is headed).

  - All these displays are interfaced using BCD to Seven Segment Display converters (7447).

- There are two sets of motors, one for the movement of the lift in upward direction, and one for the movement in the downward direction.

- There are ‘Up’ and ‘Down’ buttons outside the lift on each floor, except the third floor (which has no ‘Down’ button) and the ground floor (which has no ‘Up’ button’).

- In addition, there are four buttons (one for each floor) inside the lift to select the destination floor. Thus, there are 14 buttons in all.

- Two ‘coarse’ and one ‘fine’ sensor are used to track the movement of the elevator

Components Used

MEMORY MAPPING

RAMeven 	01000H, 01002H, 01004H, …………. 01FFEH

RAModd	01001H, 01003H, 01005H, …………. 01FFFH

ROMeven	02000H, 02002H, 02004H, …………. 02FFEH

ROModd	02001H, 02003H, 02005H, …………. 02FFFH

8086 – INTEL MICROPROCESSOR

Intel 8086 microprocessor is a first member of x86 family of processors. The 8086 has complete 16-bit architecture - 16-bit internal registers, 16-bit data bus, and 20-bit address bus (1 MB of physical memory). Because the processor has 16-bit index registers and memory pointers, it can effectively address only 64 KB of memory.

8255- INTEL Programmable Peripheral Interface chip

The Programmable Peripheral Interface (PPI) Intel 8255A is a general purpose programmable I/O device which was designed to give the CPU access to programmable parallel I/O. It provides 24 I/O pins which may be individually programmed in 2 groups of 12 and used in 3 major modes of operation.

74LS373- OCTAL D-TYPE LATCH

The 74LS343 IC consists of 8 latches with 3-state outputs for bus organized system applications. The flip-flops appear transparent to the data (data changes asynchronously) when Latch Enable (LE) is HIGH. When LE is LOW, the data that meets the setup times is latched. Data appears on the bus when the Output Enable (OE) is LOW. When OE is HIGH the bus output is in the high impedance state.

Two 74LS343 have been used in this design to de-multiplex the Address lines and interface the components in the system.

2716-(ROM-READ ONLY MEMORY)

The 2716 is a 16,384-bit EPROM organised as 2K x 8. A read-only memory is a type of non-volatile memory and is usually hard-wired. EPROM can be erased and re-programmed, but usually this can only be done at relatively slow speeds, may require special equipment to achieve, and is typically only possible a certain number of times.

Two such chips have been used in this project – one serves as the odd bank of memory and the other serves as the even bank of memory.

6116-(RAM-RANDOM ACCESS MEMORY)

The 6116 is a 16,384-bit high speed static RAM organised as 2K x 8. A random-access memory device allows data items to be read or written in almost the same amount of time irrespective of the physical location of data inside the memory. RAM also allows for faster access of data.

Two such chips have been used in this project – one serves as the odd bank of memory and the other serves as the even bank of memory.

8284 CLOCK GENERATOR

7-SEGMENT DISPLAY

The 7447 IC take in a BCD value (4 bits) as input and outputs the corresponding seven-segment display value. Two 7447 ICs have been used.

4 – Seven segment displays have been used (one on each floor) to display the floor where the elevator has reached.

1 – Seven segment display has been set up inside the elevator which displays the destination floor where the elevator is headed

Two such chips have been used in this project – one for interfacing the motor and sensor LEDs, and the other for interfacing the various 7 segment displays and the push buttons.

PUSH BUTTON

Each coarse and fine sensor, floor button outside and inside the elevator have been assumed to be a push button pushed by the elevator as it moves.

If not pressed, it generates a logic 0. This is being done to prevent the input from floating.

LED-LIGHT EMITTING DIODE

A light-emitting diode (LED) is a two-lead semiconductor light source. It is a p–n junction diode that emits light when activated.

HEAVY DUTY SERVO MOTOR

A servomotor is a rotary actuator or linear actuator that allows for precise control of angular or linear position, velocity and acceleration.  They are small in size but pack a big punch and are very energy-efficient.

Two heavy duty servo motors have been used in this project – one for moving the elevator in the upward direction, and the other for moving it in the downward direction.

RELAY

Printed circuit board (PCB) relays are compact relay devices used for power management in control system designs which require the relay to be mounted directly on the printed circuit board. In this projects the relays have been used to connect the two motors, and run them at different duty cycles.

Different PWM signals – each with the same frequency but different duty cycles - have been used to power the two motors. Relays have been interfaced to different ports of the 8255 in order to actuate and control the duty cycle values. Depending upon the output of the ports, different relays get selected, and thus the duty cycle given to the motors change as well.

CODE

.model tiny

.data

elevator_position db 00h	;00h,01h,02h,03h

travel_direction db 00h	;00(stop),01,10h

destination db 00h	;0fh(rest),00,01,02,03

new_destination db ?

destination_buffer db 8 dup (0fh)

buffer_up db 8 dup (0fh)

buffer_down db 8 dup (0fh)

destination_size dw 0

up_size dw 0

down_size dw 0

accel_stat db 00h

creg equ 06h

pa equ 00h

pb equ 02h

pc equ 04h

creg2 equ 16h

pa2 equ 10h

pb2 equ 12h

pc2 equ 14h

door_close db 0bbh

up_cat db 0e7h,0ebh,0edh

down_cat db 0bdh,0beh,0eeh

inlift_cat db 0d7h,0dbh,0ddh,0deh

sensor_cat db 7eh,7dh,7bh

.code

.startup

;initialization

in0:	mov al,88h

out creg,al

mov al,80h

out creg2,al

mov al,00h

out pa,al

mov al,00h

out pc2,al

;avoid key lockout

x0:	mov al,00h

out pc,al

x1:	in al, pc

and al,0f0h

cmp al,0f0h

jnz x1

;call delay_20ms

mov dx,4544d

w1:	nop

dec dx

jnz w1

;key press check

mov al,00h

out pc,al

x2:	in al,pc

and al,0f0h

cmp al,0f0h

jz x2

;call delay_20ms

mov dx,4544d

jf9:	nop

dec dx

jnz jf9

mov al,00h

out pc,al

in al,pc

and al,0f0h

cmp al,0f0h

jz x2

;key press identification

;column 1

mov al,0eh

mov bl,al

out pc,al

in al,pc

and al,0f0h

cmp al,0f0h

jnz x3

;column 2

mov al,0dh

mov bl,al

out pc,al

in al,pc

and al,0f0h

cmp al,0f0h

jnz x3

;column 3

mov al,0bh

mov bl,al

out pc,al

in al,pc

and al,0f0h

cmp al,0f0h

jnz x3

;column 4

mov al,07h

mov bl,al

out pc,al

in al,pc

and al,0f0h

cmp al,0f0h

jz x2

;decode key

;generate hex code

x3:	or al,bl

;up key pressed

mov cx,03h

mov di,00h

x4:	cmp al,up_cat[di]

jz x11

inc di

loop x4

;down key pressed

mov cx,03h

mov di,00h

x5:	cmp al,down_cat[di]

jz x12

inc di

loop x5

;inlift key pressed

mov cx,04h

mov di,00h

x6:	cmp al,inlift_cat[di]

jz x13

inc di

loop x6

;sensor key pressed

mov cx,03h

mov di,00h

x7:	cmp al,sensor_cat[di]

jz x14

inc di

loop x7

jmp x15

;operate according to the key type pressed

;up key press operations

x11:	mov dl,travel_direction

cmp dl,00h

jnz up6

;if not moving

mov ax,di

cmp al,elevator_position

jz up4

mov destination,al

mov cl,4

rol al,cl

mov bl,elevator_position

or al,bl

out pa,al

;set traveling direction

mov al,destination

cmp al,elevator_position

jle up3

mov travel_direction,01h	;move elv up

mov al,01h

out pc2,al

jmp up5

up3:	mov travel_direction,10h	;move elv down

mov al,02h

out pc2,al

jmp up5

up4:	mov al,04h	;elv at pos, just open door

out pc2,al

jmp up5

;if moving

up6:	mov ax,di

mov di,00

cmp up_size,00h

jz igr1

mov cx,up_size

up1:	cmp al,buffer_up[di]

jz up5

inc di

loop up1

igr1:	mov buffer_up[di],al

inc up_size

up5:	jmp x16

;down key press operations

;if not moving

x12:	inc di

mov dl,travel_direction

cmp dl,00h

jnz dwn6

;if not moving

mov ax,di

cmp al,elevator_position

jz dwn4

mov destination,al

mov cl,4

rol al,cl

mov bl,elevator_position

or al,bl

out pa,al

;set traveling direction

mov al,destination

cmp al,elevator_position

jle dwn3

mov travel_direction,01h	;move elv up

mov al,01h

out pc2,al

jmp dwn5

dwn3:	mov travel_direction,10h	;move elv down

mov al,02h

out pc2,al

jmp dwn5

dwn4:	mov al,04h	;elv at pos, just open door

out pc2,al

jmp dwn5

;if moving

dwn6:	mov ax,di

mov di,00

cmp down_size,00h

jz igr2

mov cx,down_size

dwn1:	cmp al,buffer_down[di]

jz dwn5

inc di

loop dwn1

igr2:	mov buffer_down[di],al

inc down_size

dwn5:	jmp x16

;inlift key press operations

x13:	cmp travel_direction,00h

jz inl1

;if moving, check if floor requested is in queue

;ckeck in buffer_up

mov ax,di

mov di,00h

cmp up_size,00h

jz inl7

mov cx,up_size

inl5:	cmp al,buffer_up[di]

jz inl2

inc di

loop inl5

inl7:

;check in buffer_down

mov di,00h

cmp down_size,00h

jz inl8

mov cx,down_size

inl6:	cmp al,buffer_down[di]

jz inl2

inc di

loop inl6

inl8:

;check destination

cmp al,destination

jz inl2

;check in destination_buffer

mov ax,di

mov di,00

cmp destination_size,00h

jz igr3

mov cx,destination_size

inl4:	cmp al,destination_buffer[di]

jz inl2

inc di

loop inl4

igr3:	mov destination_buffer[di],al

inc destination_size

jmp inl2

inl1:	mov ax,di

cmp al,elevator_position

jz inl2

cmp al,elevator_position

jg inl3

mov travel_direction,10h

mov destination,al

jmp inl2

inl3:	mov travel_direction,01h

mov destination,al

inl2:	jmp x16

;sensor key press operations

x14:	cmp di,00h

jnz co1

;coarse sensor 1 pressed

;check travelling direction

cmp travel_direction,01h

jnz goDown

;if travelling up

;light coarse 1 led

mov al,01h

out pb2,al

;check acceleration status

cmp accel_stat,00h

jnz co3

;if not accelerating

;close 20% generator

mov al,00h

out pc2,al

;start 30% generator

;mov al,01h

;out pa2,al

;wait

mov dx,56800d

jf8:	nop

dec dx

jnz jf8

;start 40% generator

mov al,04h

out pa2,al

;wait

mov dx,56800d

jf7:	nop

dec dx

jnz jf7

;stop 40% generator

mov al,00h

out pa2,al

;start 60% generator

mov al,10h

out pa2,al

mov accel_stat,01h

;if travelling down

;see if needs deceleration

goDown:	mov al,elevator_position

dec al

;check with destination

cmp al,destination

jnz co4

;deceleration required

co6:	;light coarse 1 led

mov al,01h

out pb2,al

;start 40% generator

mov al,08h

out pa2,al

;wait

mov dx,56800d

jf6:	nop

dec dx

jnz jf6

;start 30% generator

;mov al,02h

;out pa2,al

;wait

mov dx,56800d

jf5:	nop

dec dx

jnz jf5

;stop 40% generator

mov al,00h

out pa2,al

;stop 30% generator

;mov al,00h

;out pa2,al

;start 20% generator

mov al,02h

out pc2,al

mov accel_stat,00h

jmp co3

;check in destination_buffer

co4:	mov di,00h

cmp destination_size,00h

jz co5

mov cx,destination_size

co7:	cmp al,destination_buffer[di]

jz co6	;if found go for deceleration

inc di

loop co7

;check in buffer_down

co5:	mov di,00h

cmp down_size,00h

jz co3

mov cx,down_size

co8:	cmp al,buffer_down[di]

jz co6	;if found go for deceleration

inc di

loop co8

co3:	jmp x16

co1:	cmp di,01h

jnz co2

;fine sensor pressed

mov al,travel_direction

cmp al,01h

jnz sen1

;if moving up

;update display

inc elevator_position

mov al,elevator_position

mov bl,destination

mov cl,4

rol bl,cl

or al,bl

out pa,al

;see if it has to stop

mov al,elevator_position

cmp al,destination

jnz alt1

;stop

mov al,04h

out pc2,al

;update destination

mov al,elevator_position

mov new_destination,al

;look in destination_buffer

mov dl,new_destination

mov di,00h

cmp destination_size,00h

jz igr4

mov cx,destination_size

p2:	cmp dl,destination_buffer[di]

jg p1

mov dl,destination_buffer[di]

mov si,di

p1:	inc di

loop p2

igr4:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz sen6	;not found in destination_buffer

;found in destination_buffer

;remove from destination_buffer

mov di,si

mov cx,destination_size

dec cx

cmp cx,00h

jz ign5

p3:	mov al,destination_buffer[di+1]

mov destination_buffer[di],al

inc di

cmp di,cx

jnz p3

ign5:	mov destination_buffer[di],0fh

dec destination_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in destination_buffer

;look in buffer_up

sen6:	mov dl,new_destination

mov di,00h

cmp up_size,00h

jz igr5

mov cx,up_size

p5:	cmp dl,buffer_up[di]

jg p4

mov dl,buffer_up[di]

mov si,di

p4:	inc di

loop p5

igr5:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz sen7	;not found in buffer_up

;found in buffer_up

;remove from buffer_up

mov di,si

mov cx,up_size

dec cx

cmp cx,00h

jz ign6

p6:	mov al,buffer_up[di+1]

mov buffer_up[di],al

inc di

cmp di,cx

jnz p6

ign6:	mov buffer_up[di],0fh

dec up_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in buffer_up

;look in buffer_down

sen7:	mov dl,new_destination

mov di,00h

cmp down_size,00h

jz igr6

mov cx,down_size

p8:	cmp dl,buffer_down[di]

jg p7

mov dl,buffer_down[di]

mov si,di

p7:	inc di

loop p8

igr6:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz sen8	;not found in buffer_down

;found in buffer_down

;remove from buffer_down

mov di,si

mov cx,down_size

dec cx

cmp cx,00h

jz ign7

p9:	mov al,buffer_down[di+1]

mov buffer_down[di],al

inc di

cmp di,cx

jnz p9

ign7:	mov buffer_down[di],0fh

dec down_size

mov al,new_destination

mov destination,al

jmp sen2

;no place up to go

sen8:	mov travel_direction,10h	;go down now

;look in destination_buffer

mov dl,new_destination

mov di,00h

cmp destination_size,00h

jz igr7

mov cx,destination_size

q2:	cmp dl,destination_buffer[di]

jl q1

mov dl,destination_buffer[di]

mov si,di

q1:	inc di

loop q2

igr7:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz ben6	;not found in destination_buffer

;found in destination_buffer

;remove from destination_buffer

mov di,si

mov cx,destination_size

dec cx

cmp cx,00h

jz ign8

q3:	mov al,destination_buffer[di+1]

mov destination_buffer[di],al

inc di

cmp di,cx

jnz q3

ign8:	mov destination_buffer[di],0fh

dec destination_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in destination_buffer

;look in buffer_up

ben6:	mov dl,new_destination

mov di,00h

cmp up_size,00h

jz igr8

mov cx,up_size

q5:	cmp dl,buffer_up[di]

jl q4

mov dl,buffer_up[di]

mov si,di

q4:	inc di

loop q5

igr8:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz ben7	;not found in buffer_up

;found in buffer_up

;remove from buffer_up

mov di,si

mov cx,up_size

dec cx

cmp cx,00h

jz ign9

q6:	mov al,buffer_up[di+1]

mov buffer_up[di],al

inc di

cmp di,cx

jnz q6

ign9:	mov buffer_up[di],0fh

dec up_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in buffer_up

;look in buffer_down

ben7:	mov dl,new_destination

mov di,00h

cmp down_size,00h

jz igr9

mov cx,down_size

q8:	cmp dl,buffer_down[di]

jl q7

mov dl,buffer_down[di]

mov si,di

q7:	inc di

loop q8

igr9:	mov new_destination,dl

mov al,elevator_position

cmp al,new_destination

jz ben8	;not found in buffer_down

;found in buffer_down

;remove from buffer_down

mov di,si

mov cx,down_size

dec cx

cmp cx,00h

jz ign10

q9:	mov al,buffer_down[di+1]

mov buffer_down[di],al

inc di

cmp di,cx

jnz q9

ign10:	mov buffer_down[di],0fh

dec down_size

mov al,new_destination

mov destination,al

jmp sen2

;nowhere to go, rest

ben8:	mov al,new_destination

mov destination,al

mov travel_direction,00h

jmp sen2

;look in buffs

;look in destination_buffer

alt1:	mov al,elevator_position

mov di,00h

cmp destination_size,00h

jz igr10

mov cx,destination_size

alt3:	cmp al,destination_buffer[di]

jz alt2

inc di

loop alt3

igr10:	jmp alt5

;found it? remove

alt2:	mov cx,destination_size

dec cx

cmp cx,00h

jz ign1

alt4:	mov al,destination_buffer[di+1]

mov destination_buffer[di],al

inc di

cmp di,cx

jnz alt4

ign1:	mov destination_buffer[di],0fh

dec destination_size

mov al,04h

out pc2,al

jmp sen2

;look in buffer_up

alt5:	mov al,elevator_position

mov di,00h

cmp up_size,00h

jz igr11

mov cx,up_size

alt7:	cmp al,buffer_up[di]

jz alt6

inc di

loop alt7

igr11:	jmp sen2

;found it? remove

alt6:	mov cx,up_size

dec cx

cmp cx,00h

jz ign2

alt8:	mov al,buffer_up[di+1]

mov buffer_up[di],al

inc di

cmp di,cx

jnz alt8

ign2:	mov buffer_up[di],0fh

dec up_size

mov al,04h

out pc2,al

jmp sen2

sen1:

;if moving down

;update display

dec elevator_position

mov al,elevator_position

mov bl,destination

mov cl,4

rol bl,cl

or al,bl

out pa,al

;see if it has to stop

mov al,elevator_position

cmp al,destination

jnz act1

;stop

mov al,04h

out pc2,al

;update destination

mov al,elevator_position

mov new_destination,al

;look in destination_buffer

mov dl,new_destination

mov di,00h

cmp destination_size,00h

jz igr13

mov cx,destination_size

m2:	cmp dl,destination_buffer[di]

jl m1

mov dl,destination_buffer[di]

mov si,di

m1:	inc di

loop m2

igr13:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz sen3	;not found in destination_buffer

;found in destination_buffer

;remove from destination_buffer

mov di,si

mov cx,destination_size

dec cx

cmp cx,00h

jz ign11

m3:	mov al,destination_buffer[di+1]

mov destination_buffer[di],al

inc di

cmp di,cx

jnz m3

ign11:	mov destination_buffer[di],0fh

dec destination_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in destination_buffer

;look in buffer_up

sen3:	mov dl,new_destination

mov di,00h

cmp up_size,00h

jz igr14

mov cx,up_size

m5:	cmp dl,buffer_up[di]

jl m4

mov dl,buffer_up[di]

mov si,di

m4:	inc di

loop m5

igr14:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz sen4	;not found in buffer_up

;found in buffer_up

;remove from buffer_up

mov di,si

mov cx,up_size

dec cx

cmp cx,00h

jz ign12

m6:	mov al,buffer_up[di+1]

mov buffer_up[di],al

inc di

cmp di,cx

jnz m6

ign12:	mov buffer_up[di],0fh

dec up_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in buffer_up

;look in buffer_down

sen4:	mov dl,new_destination

mov di,00h

cmp down_size,00h

jz igr15

mov cx,down_size

m8:	cmp dl,buffer_down[di]

jl m7

mov dl,buffer_down[di]

mov si,di

m7:	inc di

loop m8

igr15:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz sen5	;not found in buffer_down

;found in buffer_down

;remove from buffer_down

mov di,si

mov cx,down_size

dec cx

cmp cx,00h

jz ign13

m9:	mov al,buffer_down[di+1]

mov buffer_down[di],al

inc di

cmp di,cx

jnz m9

ign13:	mov buffer_down[di],0fh

dec down_size

mov al,new_destination

mov destination,al

jmp sen2

;no place up to go

sen5:	mov travel_direction,10h	;go down now

;look in destination_buffer

mov dl,new_destination

mov di,00h

cmp destination_size,00h

jz igr16

mov cx,destination_size

n2:	cmp dl,destination_buffer[di]

jg n1

mov dl,destination_buffer[di]

mov si,di

n1:	inc di

loop n2

igr16:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz ben3	;not found in destination_buffer

;found in destination_buffer

;remove from destination_buffer

mov di,si

mov cx,destination_size

dec cx

cmp cx,00h

jz ign14

n3:	mov al,destination_buffer[di+1]

mov destination_buffer[di],al

inc di

cmp di,cx

jnz n3

ign14:	mov destination_buffer[di],0fh

dec destination_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in destination_buffer

;look in buffer_up

ben3:	mov dl,new_destination

mov di,00h

cmp up_size,00h

jz igr17

mov cx,up_size

n5:	cmp dl,buffer_up[di]

jg n4

mov dl,buffer_up[di]

mov si,di

n4:	inc di

loop n5

igr17:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz ben4	;not found in buffer_up

;found in buffer_up

;remove from buffer_up

mov di,si

mov cx,up_size

dec cx

cmp cx,00h

jz ign15

n6:	mov al,buffer_up[di+1]

mov buffer_up[di],al

inc di

cmp di,cx

jnz n6

ign15:	mov buffer_up[di],0fh

dec up_size

mov al,new_destination

mov destination,al

jmp sen2

;not found in buffer_up

;look in buffer_down

ben4:	mov dl,new_destination

mov di,00h

cmp down_size,00h

jz igr18

mov cx,down_size

n8:	cmp dl,buffer_down[di]

jg n7

mov dl,buffer_down[di]

mov si,di

n7:	inc di

loop n8

igr18:	mov new_destination,dl

mov al,elevator_position

cmp new_destination,al

jz ben5	;not found in buffer_down

;found in buffer_down

;remove from buffer_down

mov di,si

mov cx,down_size

dec cx

cmp cx,00h

jz ign16

n9:	mov al,buffer_down[di+1]

mov buffer_down[di],al

inc di

cmp di,cx

jnz n9

ign16:	mov buffer_down[di],0fh

dec down_size

mov al,new_destination

mov destination,al

jmp sen2

;nowhere to go, rest

ben5:	mov al,new_destination

mov destination,al

mov travel_direction,00h

jmp sen2

;look in buffs

;look in destination_buffer

act1:	mov al,elevator_position

mov di,00h

cmp destination_size,00h

jz igr19

mov cx,destination_size

act3:	cmp al,destination_buffer[di]

jz act2

inc di

loop act3

igr19:	jmp act5

;found it? remove

act2:	mov cx,destination_size

dec cx

cmp cx,00h

jz ign3

act4:	mov al,destination_buffer[di+1]

mov destination_buffer[di],al

inc di

cmp di,cx

jnz act4

ign3:	mov destination_buffer[di],0fh

dec destination_size

mov al,04h

out pc2,al

jmp sen2

;look in buffer_down

act5:	mov al,elevator_position

mov di,00h

cmp down_size,00h

jz igr21

mov cx,down_size

act8:	cmp al,buffer_down[di]

jz act6

inc di

loop act8

igr21:	jmp sen2

;found it? remove

act6:	mov cx,down_size

dec cx

cmp cx,00h

jz ign4

act7:	mov al,buffer_down[di+1]

mov buffer_down[di],al

inc di

cmp di,cx

jnz act7

ign4:	mov buffer_down[di],0fh

dec down_size

mov al,04h

out pc2,al

sen2:	jmp x16

;coarse sensor 2 pressed

;check for travelling direction

co2:	cmp travel_direction,10h

jnz goUp

;if travelling down

;light coarse 2 led

mov al,02h

out pb2,al

;check acceleration status

cmp accel_stat,00h

jnz cos3

;if not accelerating

;stop 20% generator

mov al,00h

out pc2,al

;start 30% generator

;mov al,02h

;out pa2,al

;wait

mov dx,56800d

jf1:	nop

dec dx

jnz jf1

;start 40% generator

mov al,08h

out pa2,al

;wait

mov dx,56800d

jf2:	nop

dec dx

jnz jf2

;stop 40% generator

mov al,00h

out pa2,al

;start 60% generator

mov al,20h

out pa2,al

;update acceleration status

mov accel_stat,01h

jmp cos3

;if travelling up

goUp:	mov al,elevator_position

inc al

;check with destination

cmp al,destination

jnz cos4

;deceleration required

cos6:	;light coarse 2 led

mov al,02h

out pb2,al

;start 40% generator

mov al,04h

out pa2,al

;wait

mov dx,56800d

jf3:	nop

dec dx

jnz jf3

;start 30% generator

;mov al,01h

;out pa2,al

;wait

mov dx,56800d

jf4:	nop

dec dx

jnz jf4

;stop 30% generator

;mov al,00h

;out pa2,al

;stop 40% generator

mov al,00h

out pa2,al

;start 20% generator

mov al,01h

out pc2,al

mov accel_stat,00h

jmp cos3

;check in destination_buffer

cos4:	mov di,00h

cmp destination_size,00h

jz cos5

mov cx,destination_size

cos7:	cmp al,destination_buffer[di]

jz cos6	;if found go for deceleration

inc di

loop cos7

;check in buffer_up

cos5:	mov di,00h

cmp up_size,00h

jz cos3

mov cx,up_size

cos8:	cmp al,buffer_up[di]

jz cos6	;if found go for deceleration

inc di

loop cos8

cos3:	jmp x16

;door close key press operations

x15:

cmp travel_direction,00h

jz dor1

cmp travel_direction,01h

jz dor3

mov al,destination

mov cl,4

rol al,cl

mov bl,elevator_position

or al,bl

out pa,al

mov al,02h

out pc2,al

jmp x16

dor3:	mov al,destination

mov cl,4

rol al,cl

mov bl,elevator_position

or al,bl

out pa,al

mov al,01h

out pc2,al

jmp x16

dor1:	mov destination,00h

mov elevator_position,00h

mov travel_direction,00h

jmp in0

;mov al,elevator_position

;cmp al,destination

;jz dor2

;mov al,02h

;out pc2,al

;mov al,00h

;mov cl,4

;rol al,cl

;mov bl,elevator_position

;or al,bl

;out pa,al

;jmp x16

dor2:	mov al,04h

out pc2,al

x16:	jmp x0

.exit

End

DESIGN